Current Date: 9/30/2026 9:23 AM

Units system: SI

File name: X:\Design_Dept\2026\01. QUOTATION\344-09262\REV01\02.MODEL\RAM\4026 (344-09262)_RAM CONNECTION.rcnx

Steel Connections

Results

Connection:  11 - MEP_KNEE_BCF_PERPENDICULAR_EU_1/4_PL_2B_1B1/2

Family:	Beam - Column flange (BCF)

Type:	Knee moment end plate

Design code: AISC 360-16 LRFD

Demands

Beam			Right beam			Left beam			Column			Panel

Description	Ru	Pu	Mu	PufTop	PufBot	PufTop	PufBot	Pu	Vu	Load type

[kN]	[kN]	[kN*m]	[kN]	[kN]	[kN]	[kN]	[kN]	[kN]

M_max	32.01	-3.57	145.16	-240.55	236.98	0.00	0.00	0.00	240.55	Design

M_min	-42.97	28.29	-183.09	315.30	-287.01	0.00	0.00	0.00	315.30	Design

P_max	-53.78	2.55	-112.62	186.52	-183.97	0.00	0.00	0.00	186.52	Design

P_min	-12.83	-25.81	1.66	-15.64	-10.18	0.00	0.00	0.00	15.64	Design

V2_max	26.16	37.97	-92.86	171.73	-133.75	0.00	0.00	0.00	171.73	Design

V3_max	-12.99	1.38	11.88	-18.85	20.23	0.00	0.00	0.00	20.23	Design

Geometric Considerations

Dimensions	Unit	Value	Min.	Max.	Sta.	References

Extended end plate

Vertical edge distance	[mm]	50.00	30.35	152.40		Sec. J3.5

Horizontal edge distance	[mm]	50.00	30.35	152.40		Sec. J3.5

Vertical bolt spacing (external flange)	[mm]	108.04	64.00	--		Sec. J3.3

Horizontal center-to-center spacing (gage)	[mm]	125.00	64.00	150.00		DG4 Sec. 2.1,

2.4,

DG16 Sec. 2.5

Outer bolt distance (external flange)	[mm]	50.00	36.70	--		DG4 Sec. 2.1

Inner bolt distance (external flange)	[mm]	50.00	36.70	--		DG4 Sec. 2.1

Inner bolt distance (internal flange)	[mm]	50.00	36.70	--		DG4 Sec. 2.1

Bolt diameter	[mm]	24.00	--	38.10		DG4 Sec. 1.1

Beam

Weld size (internal flange)	[1/16in]	8	3	--		table J2.4

Web	[1/16in]	8	2	--		table J2.4

Support

Horizontal edge distance	[mm]	50.00	30.35	152.40		Sec. J3.5

Transverse stiffeners

Length	[mm]	586.00	292.00	584.00		Sec. J10.8

Width	[mm]	82.50	71.00	--		Sec. J10.8

Weld size	[1/16in]	8	8	--		DG 13 Eq. 4.3-6

Plate / Column Behavior

End plate behavior (external flange)

Thin plate behavior controlled by plate yielding

End plate behavior (internal flange)

Thin plate behavior controlled by plate yielding

Connection plate behavior (external flange)

Thin plate behavior controlled by plate yielding

Connection plate behavior (internal flange)

Thin plate behavior controlled by plate yielding

Design Check

Verification	Unit	Capacity	Demand	Ctrl EQ	Ratio	References

Moment end plate (external flange)

Flexural yielding	[KN*m]	307.72	191.69	M_min	0.62	DG16 Sec 2.5

No prying bolt moment strength	[KN*m]	512.00	191.69	M_min	0.37	DG16 Sec 2.5

Bolt rupture with prying moment strength	[KN*m]	420.75	191.69	M_min	0.46	DG16 Sec 2.5

Bolts shear	[KN]	505.30	32.01	M_max	0.06	Tables (7-1..14)

Bolt bearing under shear load	[KN]	1142.61	53.78	P_max	0.05	Eq. J3-6

Shear yielding	[KN]	670.68	157.65	M_min	0.24	DG4 Eq. 3.12

Shear rupture	[KN]	566.74	157.65	M_min	0.28	DG4 Eq. 3.13

Moment end plate (internal flange)

Flexural yielding	[KN*m]	174.68	144.07	M_max	0.82	DG16 Sec 2.5

No prying bolt moment strength	[KN*m]	233.25	144.07	M_max	0.62	DG16 Sec 2.5

Bolt rupture with prying moment strength	[KN*m]	191.68	144.07	M_max	0.75	DG16 Sec 2.5

Bolts shear	[KN]	252.65	53.78	P_max	0.21	Tables (7-1..14)

Bolt bearing under shear load	[KN]	492.88	32.01	M_max	0.06	Eq. J3-6

Shear yielding	[KN]	670.68	143.51	M_min	0.21	DG4 Eq. 3.12

Beam

Web weld shear strength	[KN]	1176.05	32.01	M_max	0.03	Eq. J2-4

Web weld strength to reach yield stress	[KN/m]	5836.60	1552.50	M_max	0.27	Eq. J4-1

Shear yielding	[KN]	637.56	32.01	M_max	0.05	Eq. J4-3

Flange weld capacity (internal flange)	[KN]	863.04	236.98	M_max	0.27	Eq. J2-4

Support

Flexural yielding (external flange)	[KN*m]	353.83	191.69	M_min	0.54	DG4 Eq. 3.21

Bolt rupture with prying moment strength	[KN*m]	403.29	191.69	M_min	0.48	DG16 Sec 2.5

Support bolt bearing (external flange)	[KN]	1299.46	53.78	P_max	0.04	Eq. J3-6

Flexural yielding (internal flange)	[KN*m]	260.08	144.07	M_max	0.55	DG4 Eq. 3.21

Bolt rupture with prying moment strength	[KN*m]	174.22	144.07	M_max	0.83	DG16 Sec 2.5

Support bolt bearing (internal flange)	[KN]	649.73	32.01	M_max	0.05	Eq. J3-6

Panel web shear	[KN]	918.09	315.30	M_min	0.34	Sec. G1

Support - right side

Local web yielding	[KN]	1095.72	315.30	M_min	0.29	Sec. J10

Transverse stiffeners - bottom

Yielding strength due to axial load	[KN]	720.36	0.00	M_max	0.00	Eq. J4-1

Compression	[KN]	444.40	0.00	M_max	0.00	Sec. J4.4

Flange weld capacity	[KN]	594.48	0.00	M_max	0.00	Eq. J2-4

Web weld capacity	[KN]	3569.53	0.00	M_max	0.00	Eq. J2-4

Global critical strength ratio	0.83